# Runner SBI vs Proposed / ACT T-SBI

| Area | Current Runner SBI | Proposed / ACT T-SBI |
| --- | --- | --- |
| Purpose | Lightweight private bridge for this runner's ACT S/U execution path | Standard lightweight test SBI for ACT certification across conforming or custom M-mode |
| Full OpenSBI? | No | No |
| ABI compatibility with ratified SBI | Separate private ABI plus limited ratified SBI handling | Test-focused ABI; should coexist carefully with ratified SBI if both are present |
| Main dispatch register | a7 selects private extension, a0 selects operation | a0 selects operation directly |
| Extension ID | a7 = RUNNER_TEST_SBI_EXT = 0x56524632 | No private a7 extension in the original T-SBI table |
| Function ID register | Not using ratified a6 for private bridge | Not using a6 in original lightweight ABI |
| Basic call shape | a7 = ext, a0 = op, a1/a2 = args | a0 = op, a1 = optional arg, return in a0 |
| ECALL test op | RUNNER_TEST_OP_ECALL_TEST = 0 | ECALL_TEST = 0x00000073 in Table 6, or 0 in later system-call text |
| CSR access op | RUNNER_TEST_OP_ACCESS_CSR = 5 | Either encoded CSR instruction directly in a0 per Table 6, or a0 = 5 and a1 = opcode per later system-call text |
| CSR value argument | Uses a2 for source value when forwarding CSR access | Original Table 6 uses a1 as source value |
| CSR read result | Writes result through SBI result convention or back into saved lower trap frame | Returns result in a0, or modifies target GPR depending on handler model |
| Unknown op behavior | Often returns -2 / not supported | Returns -1 for reserved or invalid operation |
| Mode switch calls | Private GOTO_M_MODE = 1 and GOTO_S_MODE = 2, but currently return not-supported | ACT calls include GOTO_MMODE, GOTO_SMODE, GOTO_UMODE, and possibly VS/VU modes |
| Lower-mode return | Uses private RETURN_TO_M = 0x8000000000000000 | Not part of original T-SBI table; framework may use mode/trap return conventions |
| S-mode forwarding | S trap handler rewrites frame into private runner ABI and executes ecall to M-mode | S trap handler should handle what it can and forward unchanged T-SBI request upward |
| Illegal M-CSR access from S | Optional compatibility path converts illegal CSR trap into private ACCESS_CSR bridge | Expected behavior is to route/emulate illegal or explicit M-CSR access through T-SBI |
| CSR decoding | Decodes general CSR instruction format and can emulate reads/writes | Should validate exact T-SBI CSR forms depending on chosen ABI |
| CSR support scope | Limited set; compatibility-safe set is narrower for S-mode CSR policy | Must cover writable M-mode CSRs that affect lower modes: mstatus, medeleg/mideleg, mie/mip, mtvec, PMP, counters, misa, menvcfg, mseccfg, etc. |
| Ratified SBI services | Has limited support for base, timer, IPI, RFNC, legacy console/shutdown | T-SBI is separate from full OpenSBI, but system may still need ratified SBI coexistence |
| Custom M-mode support | Partial: can execute/emulate selected standard machine CSRs in M-mode | Core goal: M-mode handler may translate/emulate standard test requests onto DUT-specific custom M-mode resources |
| Test body portability | Helps isolate ACT lower-mode execution, but private ABI is runner-specific | Designed so portable ACT test body avoids direct dependency on conforming M-mode |
| Current status | Working private runner bridge, conceptually similar to T-SBI | Target architecture/ABI for certification with conforming or custom M-mode |